1-14-2025

- Powered on LAZAM Ctrl-C

- Opened Terminal

- Entered ‘sudo apt install qv412’

- Entered ‘qv412’, terminal returned “command not found”

- Entered ‘sudo apt install qv4l2’

- Entered ‘qv4l2’, terminal opened V4L2 Test Bench

- Opened ‘video 0’ in device selection

- Hit ‘Start Capturing’

  - Input set to ‘Camera 1’

  - Frame Size set to 640x480

  - Frame Rate set to 30 fps

  - Capture Image Formats set to ‘YUYV 4:2:2’

  - Colorspace set to ‘Autodetect’

  - Y’CbCr/HSV Encoding set to ‘Autodetect’

  - Video Aspect Ratio set to ‘Source Width and Height’

  - Streaming Method set to ‘Memory mapped I/O’

  - Field set to ‘None’

  - Transfer Function set to ‘Autodetect’

  - Quantization set to ‘Autodetect’

  - Pixel Aspect Ratio set to ‘Autodetect’

  - Number of Buffers set to 4

  - Use Record Priority unchecked

- ArduCam successfully streamed video with minimal latency

- Closed V4L2 Test Bench

- Powered off LAZAM Ctrl-C

1-16-2025

- Powered on LAZAM Ctrl-C

- Opened Terminal

- Entered ‘git clone https://github.com/Tinker-Twins/AprilTag.git’

- Entered ‘cd ~/AprilTag’

- Entered ‘./install.sh’, failed to install due to lack of file permissions

- Entered ‘chmod +x install.sh’

- Entered ‘./install.sh’, installed successfully

- Entered ‘cd ~/AprilTag/scripts’

- Entered ‘python3 apriltag_video.py’, script successfully captured AprilTag data from preloaded video

- Closed Terminal

- Changed line 10 in python3 apriltag_video.py

  - Was: def apriltag_video(input_streams=['../media/input/single_tag.mp4', '../media/input/multiple_tags.mp4'],

  - Now: def apriltag_video(input_streams=[0],

- Opened Terminal

- Entered ‘cd ~/AprilTag/scripts’

- Entered ‘python3 apriltag_video.py’, script successfully captured AprilTag data from live ArduCam stream

  - Used image of multiple 11h36 AprilTags on a computer monitor

  - Printed 11h36 AprilTags 0-6 on white paper

- Closed Terminal

- Powered off LAZAM Ctrl-C