In this first Wireshark lab, you’ll get acquainted with Wireshark, and make some simple packet captures and observations.

The basic tool for observing the messages exchanged between executing protocol entities is called a packet sniffer. A packet sniffer captures (“sniffs”) the traffic (messages) being sent/received from/by your computer; it will also typically store and/or display the contents of the various protocol fields in these captured messages. A packet sniffer is a passive tool that monitors messages sent and received by applications and protocols on your computer without ever sending packets itself. It doesn't directly receive packets addressed to it; rather, it captures copies of packets that are transmitted to or from the applications and protocols running on your system.

Figure 1 illustrates the structure of a packet sniffer. On the right side of the figure are the protocols (in this case, Internet protocols) and applications (such as a web browser or email client) that typically run on your computer. The packet sniffer, depicted within the dashed rectangle in Figure 1, is an additional component to the usual software on your machine and consists of two main parts. The packet capture library intercepts a copy of every link-layer frame sent or received by your computer over a given interface (like Ethernet or WiFi). Messages exchanged by higher-layer protocols such as HTTP, FTP, TCP, UDP, DNS, or IP are eventually encapsulated in link-layer frames that are transmitted over physical media like an Ethernet cable or a WiFi radio. By capturing all link-layer frames, you obtain all the messages sent or received across the monitored link by all protocols and applications running on your computer.

Figure 1: packet sniffer structure

The second component of a packet sniffer is the packet analyzer, which displays the contents of all fields within a protocol message.  To do so, the packet analyzer must “understand” the structure of all messages exchanged by protocols.  For example, suppose we are interested in displaying the various fields in messages exchanged by the HTTP protocol in Figure 1. The packet analyzer understands the format of Ethernet frames, and so can identify the IP datagram within an Ethernet frame.  It also understands the IP datagram format, so that it can extract the TCP segment within the IP datagram.  Finally, it understands the HTTP protocol and so, for example, knows that the first bytes of an HTTP message will contain the string “GET,” “POST,” or “HEAD,”.

We will be using the Wireshark packet sniffer [http://www.wireshark.org/] for this lab, allowing us to display the contents of messages being sent/received from/by protocols at different levels of the protocol stack.  (Technically speaking, Wireshark is a packet analyzer that uses a packet capture library in your computer.  Also, technically speaking, Wireshark captures link-layer frames as shown in Figure 1 but uses the generic term “packet” to refer to link-layer frames, network-layer datagrams, transport-layer segments, and application-layer messages, so we’ll use the less-precise “packet” term here to go along with Wireshark convention). Wireshark is a free network protocol analyzer that runs on Windows, Mac, and Linux/Unix computers. It’s an ideal packet analyzer for our lab – it is stable, has a large user base and well-documented support that includes a user-guide (http://www.wireshark.org/docs/wsug_html_chunked/), man pages (http://www.wireshark.org/docs/man-pages/), and a detailed FAQ (http://www.wireshark.org/faq.html), rich functionality that includes the capability to analyze hundreds of protocols, and a well-designed user interface. It operates in computers using Ethernet, serial (PPP), 802.11 (WiFi) wireless LANs, and many other link-layer technologies.

Downloading Wireshark

Download and install the Wireshark software:

- Go to http://www.wireshark.org/download.html and download and install the Wireshark binary for your computer.

- To run Wireshark, you’ll need to have access to a computer that supports both Wireshark and the libpcap or WinPCap packet capture library. The libpcap software will be installed for you, if it is not installed within your operating system, when you install Wireshark.

See http://www.wireshark.org/download.html for a list of supported operating systems and download sites.

Running Wireshark

When you run the Wireshark program, you’ll get a startup screen that looks something like the screen below.  Different versions of Wireshark will have different startup screens – so don’t panic if yours doesn’t look exactly like the screen below!  The Wireshark documentation states “As Wireshark runs on many different platforms with many different window managers, different styles applied and there are different versions of the underlying GUI toolkit used, your screen might look different from the provided screenshots. But as there are no real differences in functionality these screenshots should still be well understandable.”  Well said.

Figure 2: Initial Wireshark Screen

There’s not much that’s very interesting on this screen. But note that under the Capture section, there is a list of connection types and the activity. My Windows has a few different interfaces (Virtual Machines count too) but the real one, where the traffic goes through is– “Wi-Fi,” (shaded in blue in Figure 2).  All packets to/from this computer will pass through the Wi-Fi interface, so it’s here where we’ll want to capture packets. Double click on this interface (or on another computer locate the interface on startup page through which you are getting Internet connectivity, e.g., mostly a WiFi or Ethernet interface, and select that interface).

Let’s take Wireshark out for a spin! Select the interface you want to start packet capture (i.e., for Wireshark to begin capturing all packets being sent to/from that interface), a screen like the one below will be displayed, showing information about the packets being captured.  Once you start packet capture, you can stop it by using the Capture pull down menu and selecting Stop (or by clicking on the red square button next to the Wireshark fin in Figure 3).

Figure 3: Wireshark window, during and after capture

Packet content

Details of selected packet

List of captured packets

Filter

Command menus

Figure 3B: Wireshark window, during and after capture

This looks more interesting! The Wireshark interface has five major components:

- The command menus are standard pulldown menus located at the top of the Wireshark window (and on a Mac at the top of the screen as well; the screenshot in Figure 3 is from a Mac). Look now at the File and Capture menus. The File menu allows you to save captured packet data or open a file containing previously captured packet data and exit the Wireshark application. The Capture menu allows you to begin packet capture.

- The packet-listing window displays a one-line summary for each packet captured, including the packet number (assigned by Wireshark; note that this is not a packet number contained in any protocol’s header), the time at which the packet was captured, the packet’s source and destination addresses, the protocol type, and protocol-specific information contained in the packet. The packet listing can be sorted according to any of these categories by clicking on a column name.  The protocol type field lists the highest-level protocol that sent or received this packet, i.e., the protocol that is the source or ultimate sink for this packet.

- The packet-header details window provides details about the packet selected (highlighted) in the packet-listing window.  (To select a packet in the packet-listing window, place the cursor over the packet’s one-line summary in the packet-listing window and click with the left mouse button.).  These details include information about the Ethernet frame (assuming the packet was sent/received over an Ethernet interface) and IP datagram that contains this packet. The amount of Ethernet and IP-layer detail displayed can be expanded or minimized by clicking on the plus/minus boxes or right/downward-pointing triangles to the left of the Ethernet frame or IP datagram line in the packet details window.  If the packet has been carried over TCP or UDP, TCP or UDP details will also be displayed, which can similarly be expanded or minimized.  Finally, details about the highest-level protocol that sent or received this packet are also provided.

- The packet-contents window displays the entire contents of the captured frame, in both ASCII and hexadecimal format.

- Towards the top of the Wireshark graphical user interface, is the packet display filter field, into which a protocol name or other information can be entered in order to filter the information displayed in the packet-listing window (and hence the packet-header and packet-contents windows).  In the example below, we’ll use the packet-display filter field to have Wireshark hide (not display) packets except those that correspond to HTTP messages.

Testing Wireshark

The best way to learn about any new piece of software is to try it out!  We’ll assume that your computer is connected to the Internet via wireless 802.11 WiFi interface. Do the following:

- Start up your favorite web browser, which will display your selected homepage.

- Start up the Wireshark software.  You will initially see a window similar to that shown in Figure 2. Wireshark has not yet begun capturing packets.

- To begin packet capture, select the Capture pull down menu and select Interfaces. This will cause the “Wireshark: Capture Options” window to be displayed and select “Manage Interfaces”.  You should see a list of interfaces, as shown in Figures 4a and 4b (Windows) and 4c (Mac).

Figure 4a: Wireshark Capture Options window, on a Windows computer

Figure 4b: Wireshark Manage interface window, on a Windows computer

| Figure 4c: Wireshark Capture interface window, on a Mac computer |  |  |
| --- | --- | --- |

- You’ll see a list of the interfaces on your computer as well as a count of the packets that have been observed on that interface so far.  On a Windows machine, click on Start for the interface on which you want to begin packet capture (in the case in Figure 4a, the Gigabit network Connection).  On a Windows machine, select the interface and click Start on the bottom of the window). Packet capture will now begin - Wireshark is now capturing all packets being sent/received from/by your computer!

- Once you begin packet capture, a window similar to that shown in Figure 3 will appear.  This window shows the packets being captured.  By selecting Capture pulldown menu and selecting Stop, or by click on the red Stop square, you can stop packet capture.   But don’t stop packet capture yet.  Let’s capture some interesting packets first.  To do so, we’ll need to generate some network traffic.  Let’s do so using a web browser, which will use the HTTP protocol that we will study in detail in class to download content from a website.

- While Wireshark is running, enter the URL: 
http://gaia.cs.umass.edu/wireshark-labs/INTRO-wireshark-file1.html
and have that page displayed in your browser. In order to display this page, your browser will contact the HTTP server at gaia.cs.umass.edu and exchange HTTP messages with the server in order to download this page. The Ethernet or WiFi frames containing these HTTP messages (as well as all other frames passing through your Ethernet or WiFi adapter) will be captured by Wireshark.

- After your browser has displayed the INTRO-wireshark-file1.html page (it is a simple one line of congratulations), stop Wireshark packet capture by selecting stop in the Wireshark capture window. The main Wireshark window should now look similar to Figure 3. You now have live packet data that contains all protocol messages exchanged between your computer and other network entities!  The HTTP message exchanges with the gaia.cs.umass.edu web server should appear somewhere in the listing of packets captured.  But there will be many other types of packets displayed as well (see, e.g., the many different protocol types shown in the Protocol column in Figure 3).  Even though the only action you took was to download a web page, there were evidently many other protocols running on your computer that are unseen by the user.  We’ll learn much more about these protocols as we progress through the text!  For now, you should just be aware that there is often much more going on than “meet’s the eye”!

- Type in “http” or “tcp.port == 80” (without the quotes, and in lower case – all protocol names are in lower case in Wireshark, and make sure to press the enter/return key) into the display filter specification window at the top of the main Wireshark window.  Then select Apply (to the right of where you entered “http”) or just hit return.  This will cause only HTTP message to be displayed in the packet-listing window. Figure 5 below shows a screenshot after the http filter has been applied to the packet capture window shown earlier in Figure 3.  Note also that in the Selected packet details window, we’ve chosen to show detailed content for the Hypertext Transfer Protocol application message that was found within the TCP segment, that was inside the IPv4 datagram that was inside the Ethernet II (WiFi) frame.  Focusing on content at a specific message, segment, datagram and frame level lets us focus on just what we want to look at (in this case HTTP messages).

Figure 5: looking at the details of the HTTP message that contained a GET of http://gaia.cs.umass.edu/wireshark-labs/INTRO-wireshark-file1.html

- If you don’t see any GET message do the following:

The likely problem is that your browser is upgrading the HTTP request to HTTPS, so Wireshark doesn't show a plain HTTP GET.

For your Wireshark lab, the easiest solution is to generate the request outside the browser.

-Start capturing on Wi-Fi in Wireshark.

-Open Command Prompt.

-Run: curl --http1.1 http://gaia.cs.umass.edu/wireshark-labs/INTRO-wireshark-ile1.html

-Return to Wireshark.

-Enter this filter: http.request.method == "GET"/http

- Find the HTTP GET message that was sent from your computer to the gaia.cs.umass.edu HTTP server. (Look for an HTTP GET message in the “listing of captured packets” portion of the Wireshark window (see Figures 3 and 5) that shows “GET” followed by the gaia.cs.umass.edu URL that you entered.  When you select the HTTP GET message, the Ethernet frame, IP datagram, TCP segment, and HTTP message header information will be displayed in the packet-header window. By clicking on ‘+’ and ‘-' and right-pointing and down-pointing arrowheads to the left side of the packet details window, minimize the amount of Frame, Ethernet, Internet Protocol, and Transmission Control Protocol information displayed.  Maximize the amount information displayed about the HTTP protocol.  Your Wireshark display should now look roughly as shown in Figure 5. (Note, in particular, the minimized amount of protocol information for all protocols except HTTP, and the maximized amount of protocol information for HTTP in the packet-header window).

- Exit Wireshark

Congratulations!  You’ve now completed the first lab! The following are a few examples of questions I could ask you. You don’t have to answer them if you want to for this lab.

- Which of the following protocols are shown as appearing (i.e., are listed in the Wireshark “protocol” column) in your trace file: TCP, QUIC, HTTP, DNS, UDP, TLSv1.2?

- How long did it take from when the HTTP GET message was sent until the HTTP OK reply was received for the link provided in step 6? (By default, the value of the Time column in the packet-listing window is the amount of time, in seconds, since Wireshark tracing began.  (If you want to display the Time field in time-of-day format, select the Wireshark View pull down menu, then select Time Display Format, then select Time-of-day.)

- What is the Internet address of the gaia.cs.umass.edu (also known as www-net.cs.umass.edu)?  What is the Internet address of the computer that sent the HTTP GET message?

To answer the following two questions, you’ll need to select the TCP packet containing the HTTP GET request. The purpose of these next two questions is to familiarize you with using Wireshark’s “Details of selected packet window”; see Figure 3.  To do this, click on Packet.  To answer the first question below, then look in the “Details of selected packet” window toggle the triangle for HTTP (your screen should then look like Figure 5); for the second question below, you’ll need to expand the information on the Transmission Control Protocol (TCP) part of this packet.

- Expand the information on the Transmission Control Protocol for this packet in the Wireshark “Details of selected packet” window (see Figure 3 in the lab writeup) so you can see the fields in the TCP segment carrying the HTTP message. What is the destination port number (the number following “Dest Port:” for the TCP segment containing the HTTP request) to which this HTTP request is being sent?

- Print the two HTTP messages (GET and OK) referred to in question 2 above. To do so, select Print from the Wireshark File command menu, and select the “Selected Packet Only” and “Print as displayed” radial buttons, and then click OK.

- It is possible to access other people’s navigation using Wireshark when you access a public network. Why do you have to be extra thoughtful regarding what services you access while navigating using a public network? Give an example.