SYS 304: Trade Studies, Risk, and Decision Analysis

Simulation Approaches for Decision Making

A Reliability Problem

A milling machine has three different bearings that fail in service. The distribution of the life of each bearing is identical (Table 1). When a bearing fails, the mill stops, a repairperson is called, and a new bearing is installed. The delay time of the repairperson’s arriving at the milling machine is also a random variable having the distribution given in Table 2. Downtime for the mill is estimated at $10 per minute. The direct on-site cost of the repairperson is $30 per hour. It takes 20 minutes to change one bearing, 30 minutes to change two bearings, and 40 minutes to change three bearings. A proposal has been made to replace all three bearings whenever a bearing fails. Management needs an evaluation of the proposal. The total cost per 10,000 bearing-hours will be used as the measure of performance.

| Bearing Life (hours) | Probability | Cumulative Probability |
| --- | --- | --- |
| 1000 | 0.10 | 0.10 |
| 1100 | 0.13 | 0.23 |
| 1200 | 0.25 | 0.48 |
| 1300 | 0.13 | 0.61 |
| 1400 | 0.09 | 0.70 |
| 1500 | 0.12 | 0.82 |
| 1600 | 0.02 | 0.84 |
| 1700 | 0.06 | 0.90 |
| 1800 | 0.05 | 0.95 |
| 1900 | 0.05 | 1.00 |

| Delay Time (Minutes) | Probability | Cumulative Probability |
| --- | --- | --- |
| 5 | 0.6 | 0.6 |
| 10 | 0.3 | 0.9 |
| 15 | 0.1 | 1.0 |

Table 3 represents a simulation of 15 bearing changes under the current method of operation. Note that there are instances where more than one bearing fails at the same time. This is unlikely to occur in practice and is due to using a rather coarse grid of 100 hours for bearing life. It will be assumed in this example that the times are never exactly the same and thus no more than one bearing is changed at any breakdown.

|  | Bearing 1 |  | Bearing 2 |  | Bearing 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Life (hours) | Delay (min) | Life (hours) | Delay (min) | Life (hours) | Delay (min) |
| 1 | 1400 | 10 | 1500 | 10 | 1100 | 5 |
| 2 | 1300 | 5 | 1100 | 5 | 1100 | 5 |
| 3 | 1900 | 5 | 1500 | 5 | 1400 | 5 |
| 4 | 1500 | 5 | 1700 | 5 | 1000 | 10 |
| 5 | 1300 | 5 | 1800 | 15 | 1300 | 10 |
| 6 | 1400 | 10 | 1500 | 5 | 1100 | 5 |
| 7 | 1500 | 5 | 1000 | 10 | 1100 | 5 |
| 8 | 1800 | 10 | 1100 | 10 | 1000 | 10 |
| 9 | 1200 | 15 | 1100 | 5 | 1300 | 5 |
| 10 | 1000 | 5 | 1100 | 5 | 1600 | 15 |
| 11 | 1900 | 10 | 1000 | 5 | 1100 | 5 |
| 12 | 1400 | 5 | 1000 | 15 | 1200 | 5 |
| 13 | 1300 | 10 | 1200 | 5 | 1700 | 10 |
| 14 | 1700 | 5 | 1000 | 5 | 1400 | 5 |
| 15 | 1700 | 5 | 1100 | 5 | 1200 | 5 |
| Total | 22,300 | 110 | 18,700 | 110 | 18,600 | 105 |

The cost of the current system is estimated as follows:

Cost of bearing = 45 bearings * $32/bearing = $1,440

Cost of delay time = (110 + 110 + 105) minutes * $10/minute = $3,250

Cost of downtime during repair = 45 bearings * 20 minutes/bearing * $10/minute = $9,000

Cost of repairpersons = 45 bearings * 20 minutes/bearing * $30/60minutes = $450

Total cost = $1,440+$3,250+$9,000+$450 = $14,140

The total life of the bearings is (22,300 + 18,700 +18,600) = $59,600 hours. Therefore, the total cost per 10,000 bearing-hours is ($14,140/5.96) = $2,372.

Table 4 is a simulation of the proposed method. Note that the random digits are not shown. For the first set of bearings, the earliest failure is at 1,000 hours. All three bearings are replaced at that time, even though the remaining bearings had more life in them. For example, Bearing 1 would have lasted 700 additional hours.

The cost of the proposed system is estimated as follows:

Cost of bearings = 45 bearings * $32/bearing = $1,440

Cost of delay time = 110 minutes * $10/minute = $1,100

Cost of downtime during repair = 15 sets * 40 minutes/set * $10/minute = $6,000

Cost of repairpersons = 15 sets * 40 minutes/bearing * $30/60minutes = $300

Total cost = $1,440+$1,100+$6,000+$300 = $8,840

|  | Bearing 1 Life (hours) | Bearing 2 Life (hours) | Bearing 3 Life (hours) | First Failure (hours) | Delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1700 | 1100 | 1000 | 1000 | 10 |
| 2 | 1000 | 1800 | 1200 | 1000 | 5 |
| 3 | 1500 | 1700 | 1300 | 1300 | 5 |
| 4 | 1300 | 1100 | 1800 | 1100 | 5 |
| 5 | 1200 | 1100 | 1300 | 1100 | 5 |
| 6 | 1000 | 1200 | 1200 | 1000 | 10 |
| 7 | 1500 | 1700 | 1200 | 1200 | 5 |
| 8 | 1300 | 1700 | 1000 | 1000 | 10 |
| 9 | 1800 | 1200 | 1100 | 1100 | 15 |
| 10 | 1300 | 1300 | 1100 | 1100 | 5 |
| 11 | 1400 | 1300 | 1900 | 1300 | 10 |
| 12 | 1500 | 1300 | 1400 | 1300 | 5 |
| 13 | 1500 | 1800 | 1200 | 1200 | 10 |
| 14 | 1000 | 1900 | 1400 | 1000 | 5 |
| 15 | 1300 | 1700 | 1700 | 1300 | 5 |
| Total |  |  |  | 17,000 | 110 |

The total life of the bearings is ($17,000*3) = 51,000 hours. Therefore, the total cost per 10,000 bearing-hours is ($8,840/5.1) = $1,733.

The new policy generates a savings of $634 per 10,000 hours of bearing-life. If the machine runs continuously, the savings are about $556 per year.